Hybrid Off Grid Solar Inverter

- Rated power : 1KW -5.5KW

- Pure sine wave solar inverter

- Output power factor 1

- High PV input voltage range

- Built-in 30/60/ 80A MPPT solar charger

- Battery equalization function to optimize battery performance and extend life cycle

- Built-in anti-dusk kit for harsh environment

Features

BG Series (1KW-5.5KW)

Introduction

BG Series is a multi-functional inverter/charger, combining functions of inverter, solar charger and battery charger to offer uninterruptible power support in portable size. Its comprehensive LCD display offers user-configurable and easy-accessible button operation such as battery charging current, AC/solar charger priority, and acceptable input voltage based on different applications.

| MODEL |  | BG1000 | BG-2000 | BG-3000 | BG-4000 | BG5000 | BG5500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Battery System Voltage |  | 12/24VDC |  | 24/48VDC |  |  |  |
| INVERTER OUTPUT | Rated Power | 1000W | 2000W | 3000W | 4000W | 5000W | 5500W |
|  | Surge Power | 2000W | 4000W | 6000W | 8000W | 10000W | 11000W |
|  | Waveform | Pure Sine Wave |  |  |  |  |  |
|  | AC Voltage Regulation (Batt.Mode) | (220VAC~240VAC)±5% |  |  |  |  |  |
|  | Inverter Efficiency(Peak) | 93% |  |  |  |  |  |
|  | Transfer Time | 10ms (For Personal Computers) 20ms (For Home Appliance) |  |  |  |  |  |
| AC INPUT | Voltage | 230VAC |  |  |  |  |  |
|  | Selectable Voltage Range | 170~280VAC(For pensonal computer) \ 90~280VAC(For Home Appliance) \ 184~253VAC(VDE4105) |  |  |  |  |  |
|  | Frequency Range | 50Hz/60Hz(Auto sensing) |  |  |  |  |  |
| BATTERY | Normal Voltage | 24VDC |  | 48VDC |  |  |  |
|  | Floating Charge Voltage | 27.4VDC |  | 54.8VDC |  |  |  |
|  | Overcharge Protection | 30VDC |  | 60VDC |  |  |  |
| SOLAR CHARGER & AC CHARGER | Maximum PV Array Open Circuit Voltage | 145VDC |  |  |  |  |  |
|  | PV Array MPPT Voltage Range | 30~130VDC |  | 64~130VDC |  |  |  |
|  | Standby Power Consumption | 2W |  |  |  |  |  |
|  | PV Input Power | 2000W |  | 4000W |  |  |  |
|  | Maximum Solar Charge Current | 30/60/80A |  |  |  |  |  |
|  | Maximum Effciency | 98% |  |  |  |  |  |
|  | Maximum AC Charge Current | 20A |  | 20A |  |  |  |
|  | Maximum Charge Current | 80A |  | 100A |  |  |  |
| MECHANICAL SPECIFICATIONS | Machine Dimensions(W*H*D)(mm) | 380*266*153 |  | 380*600*153 |  |  |  |
|  | Package Dimensions(W*H*D)(mm) | 449*358*230 |  | 449*680*230 |  |  |  |
|  | Net Weight(kg) | 10 | 11 | 13.3 |  |  |  |
|  | Gross Weight(kg) | 11.7 | 12 | 16.4 |  |  |  |
| OTHER | Humidity | 5% to 95% Relatiy Humidity (Non-condensing) |  |  |  |  |  |
|  | Operating Temperature | 0°C ~55°C |  |  |  |  |  |
|  | Storage Temperature | -15°C ~60°C |  |  |  |  |  |

Specification

Back panel printing description	Solar system connection

- Input Voltage

- Working  Model

SOLAR PANELS

LAPTOP

LIGHTING

AC OUTPUT

PRINTER

INVERTER

TV

AC INPUT

AIR CONDITIONING

GENERATOR

BATTERY

Output Voltage

- Solar Panel

- Battery Status

- Loading Application

- Silent Model

- Loading Status